| Course: | HD in Artificial Intelligence and Robotics |  | Official Use |
| --- | --- | --- | --- |
| Course code: | EG114728 |  |  |
| Module: | Industrial Automation |  |  |
| Module code: | MBS4521 |  |  |

## Lab 5:

## Programmable Logic Controller II – Controlling Pneumatic circuits

| Student Name: | Cheung Tsz Chun Noddy |  | Notes to Students: - Maximum of 4 members per group for software/hardware development - Submit this lab sheet in pen writing - Individual submission required - Use spaces given for each part - Due date will be given in lab session - A maximum of 5% will be deducted for untidiness - Late submission will normally not be accepted |
| --- | --- | --- | --- |
| Student number: | 220171174 |  |  |
| Names of other members: | 1. 2. 3. |  |  |
| Date received: Due date: | 1/4 |  |  |
| Signature: | Noddy |  |  |

Objective

After completion of this lab, students should be able to:

1.	design sequential control pneumatic circuit ladder with a PLC software.

2.	build and debug PLC pneumatic circuits and test with PLC ladder.

Introduction

With previous experience in PLC ladder design with simple I/O circuits, students should be able to control more real-world systems with different type of sensors and actuators. In this session, students will be experiencing how to migrate the hard-wiring electro-pneumatic circuit control to PLC control of the same circuit operation. Also, students will be control DC motors with direction control.

Solenoid

Internal structure

Fig. 1 Internal Structure of a Solenoid

Two basic configuration: “Pull-type” as it pulls the connected load towards itself when energized, and the “Push-type” that act in the opposite direction pushing it away from itself when energized.

Solenoid door lock

Solenoids applications

Solenoid Valve

Double Solenoid Pneumatic Control Valve

Fig. 2 Components built with Solenoids

Timer-delay relay

Fig.3 shows an example of a timer relay. Time-delay relay contacts must be specified not only as either normally-open or normally-closed, but whether the delay operates in the direction of closing or in the direction of opening.

Fig.3 A timer relay

A special class of electromechanical relays called time-delay relays provide delayed action, either upon power-up or power-down, and are commonly denoted in ladder logic diagrams by "TD", "TR" or “T” designations near the coil symbols and arrows on the contact symbols.

Fig.4 shows an example of a time-delay relay contact used in a solenoid control circuit:

3 sec.

In this circuit, the solenoid, A+, delays start-up until three seconds after the pushbutton, PB1, is pressed, but will stop immediately when the pushbutton is released. The relay contact is referred to as normally-open, timed-closed, or NOTC. It is alternatively referred to as a normally-open, on-delay contact.

Below are both written and visual descriptions on how some of the common timing functions operate. A Timing Chart shows the relationship between Input Voltage, Trigger (if present) and Output.

Fig.5 ON DELAY and OFF DELAY explained with timing chart

In this laboratory session, students are required to work in groups (4 members maximum). Each group is required to write and design PLC ladder diagrams and connect I/O devices to test the functionality of programs. Every student should hand in his/her own lab sheet one week after the lab session.

Apparatus

1.	Programmable Logic Controller software (Panasonic NAIS FP Win GR ver. 2.0)

2.	PLC – FP Sigma (Model: FPG-C32T2H)

3.	Personal computers

4.	I/O Hub, Button-Light Box, cable and accessories

5.	Electro-pneumatic components, Pneumatic training board

6.	Sensors and motors.

Procedures

Part A – 	A double acting cylinder performing an oscillatory movement with delay operation run in PLC (20%)

A double acting cylinder is controlled by a 5/2 double solenoid actuated directional control valve with two limit switches as position sensors. A timer relay is used to delay the cylinder after it reaches its maximum extended position.

Actuation requirement:	When a START button (NO) is pressed (one shot with spring return), the cylinder will extend to its limit position, stay still for 5 seconds, and then it will return to its fully retracted position. The cycle repeats again automatically. When the STOP button is pressed, the cylinder will stop running after that cycle.

The electro-pneumatic sub-circuit and the ladder diagram are given below:

A1. Draw the I/O assignment table for PLC connection. Connected to; Input address; Output address. Pushbutton PB1 (NO); X0. Sensor a0 (NO); X1. Sensor a1 (NO); X2. Solenoid A+; Y6. Solenoid A-; Y7. Pushbutton PB2 (NO) X3

A2. Sketch the PLC wiring circuit.

A3. Redraw the electromechanical ladder into a PLC ladder.

A4.	Use the pneumatic training board and the given PLC to operate the circuit.

Official use:

Satisfactory / Minor assistance required / Major assistance required / Incomplete

Part B – 	Controlling multiple cylinders (50%)

Given:

- Two double acting cylinders,

- Two 5/2 double solenoid directional control valves,

- Four limit switches,

- one momentary pushbutton.

Connect all components to PLC and design a PLC ladder to control the motion sequence as given.

B1.	Control the below sequence with the pushbutton being pressed momentarily. Use a single cycle design, i.e. the sequence will only be run once when the pushbutton is pressed momentarily.

A+, 3 seconds, B+, 3 seconds, B-, 3 seconds, A-

Sequence:

B1.1 Complete the below I/O Assignment Table: Connected to; Input address; Output address. Pushbutton PB1 (NO); X1. Sensor a0 (NO); X2. Sensor a1 (NO); X3. Sensor b0 (NO); X4. Sensor b1 (NO); X5. Solenoid A+; Y3. Solenoid A-; Y2. Solenoid B+; Y5. Solenoid B-; Y4.

B1.2 Draw a circuit to connect the double acting cylinders to two 5/2 double solenoid valves and the PLC with four limit switches and a pushbutton.

B1.3	Use the pneumatic training board and the given PLC to operate the circuit.

Official use:

Satisfactory / Minor assistance required / Major assistance required / Incomplete

B2.	With the circuit built in B1/B2, implement the following motion sequence in single cycle mode.

Start, A+, delay 5 seconds, B+, B-, B+, B-, delay 3 seconds, A-

Sequence:

B2.1 Design a PLC ladder to achieve the required sequence.

B2.2	Use the pneumatic training board and the given PLC to operate the circuit.

Official use:

Satisfactory / Minor assistance required / Major assistance required / Incomplete