| Course: | HD in Artificial Intelligence and Robotics |  | Official Use |
| --- | --- | --- | --- |
| Course code: | EG114728 |  |  |
| Module: | Industrial Automation |  |  |
| Module code: | MBS4521 |  |  |

## Lab 3:

## Electro-pneumatic Circuits II

| Student Name: | Cheung Tsz Chun Noddy |  | Notes to Students: - Maximum of 4 members per group for software/hardware development - Submit this lab sheet in pen writing - Individual submission required - Use spaces given for each part - Glue print out in specified spaces - Due date will be given in lab session - A maximum of 5% will be deducted for untidiness - Late submission will normally not be accepted |
| --- | --- | --- | --- |
| Student number: | 220171174 |  |  |
| Names of other members: | 1.Lai Ho 2.Lai Ka Ming 3.Lui Shing Tak |  |  |
| Date received: Due date: | 1/4 , 30/4 |  |  |
| Signature: | Cheung Tsz Chun Noddy |  |  |

Objective

After completion of this lab, students should be able to:

1.	build circuits with more than two cascade groups by using double solenoid or single solenoid control valves

2.	design electrical ladder diagrams with emergency buttons.

Procedures

For a specific assembly line, the following control sequence is given:

A+, B+, A-, C+, C-, B-

A1.	Design a complete circuit using double acting cylinders and 5/2 double solenoid valves for a single cycle operation. (20%)

- Cascade groups

- Block diagram

- Displacement-time diagram

- Pneumatic sub-circuit

- Electrical ladder

A2.	Same requirement as A1 above, change the solenoid valves to 5/2 single solenoid spring return valves. (20%)

- Pneumatic sub-circuit

- Electrical ladder

A3.	Add an option of single or continuous cycle operation to A1 and A2. (10%)

- You may use another ink to identify the modification.

A3(A1):

A3(A2):

A4.	Add an emergency stop button (E-STOP) to A1 and A2, such that when the button is pressed, all cylinders will immediately return to their initial state. (10%)

- You may use another ink to identify the modification.

A4(A1):

A4(A2):

A5.	If only single acting cylinders are available, suggest how the above designs should be modified. (10%)

Part B –Discussion (30%)

- End -